Checklist

Revision 1.2

October 27th 2014

Cessna Skylane

Model 182S

LN-EDB

CHECKLIST REVISION LOG

| Rev# | Date | Revised pages | Description of revision |
| --- | --- | --- | --- |
| 1.0 | 2012/01/10 | New | New basic checklist based on POH |
| 1.1 | 2013/03/22 | 5 11 | Alternator belt not accessible for insp Open cowl flap before landing |
| 1.2 | 2014/10/27 | 9 11 | Pitot heat on GPS/NAV set to NAV for approach |

Statement:

This checklist complies with Pilot’s Operating Handbook for Cessna Model 182S with NAV II Avionics, revision 4, dated November 1st 2001.

Sameiet LN-EDB

AIRSPEED/FLIGHT LOAD SHEET

| Reg. Appl.: LN-EDB (G-IBZS) |  |
| --- | --- |
| AIRSPEED LIMITATIONS | AIRSPEEDS FOR NORMAL OPS |
| Vne; 170 KIAS. Vno; 140 KIAS. Va; 110 KIAS at max GW. 101 KIAS at 2600 lbs. 89 KIAS at 2000 lbs. Vfe; 0-10°: 140 KIAS 10-20°: 120 KIAS 20° +: 100 KIAS. Vs1; 40-43 KIAS. Vs30; 43-46 KIAS. Vs45; 48-51 KIAS. Vs60; 57-61 KIAS. Vso; 35-36 KIAS. Vxw; 15 KIAS. | Vr; 50-60 KIAS. Vx; (flaps); N/A. (no flaps); 65 KIAS*. Vy; (flaps); N/A. (no flaps); 80 KIAS*. Venroute climb; - IAS. Vbest glide 3100 lbs 2600 lbs 2000 lbs; 75 KIAS 70 KIAS 62 KIAS. Vapproach; 70-80 KIAS. Vfinal approach; 60-70 KIAS. Vfinal app short fld; 60 KIAS. |
| FLIGHT LOADS POSITIVE NEGATIVE Flaps up 3.8 G 1.52 G Flaps down 2.0 N/A |  |
| Remarks: Vx increases with altitude and GW (63 KIAS Sea Level – 66 KIAS FL 100). Vy decreases with altitude, increases with GW (80 KIAS – 72 KIAS). Vbest glide varies as Vmax range. Vturbulance as Va. Gusty T/O and Landing: Add max. 1/2 of wind gust factor to speed. *Value adjusted to nearest 0 or 5 value. |  |

1. PREFLIGHT/DI

DI Specific checks marked with *

A. GENERAL

Aircraft documents	CHECK/ON BOARD

Aircraft	GENERAL OVERVIEW

Fuel selector	BOTH

Fuel filters (10 + 2 drain valves, see note)	DRAIN

NOTE: Perform fuel draining before moving the aircraft. Draining after refueling has limited effect unless the aircraft has been parked for a while.

B. COCKPIT

Cockpit (incl. Headsets, First aid kit, Fire Extinguisher)	CHECK

Parking brake	SET

Control wheel lock	REMOVE

Magnetos	OFF

Avionics master/Electrical equipment	OFF

Master switch	ON

Fuel quantity indicators and annunciators	CHECK ENOUGH

*Avionics master switch	ON

*Avionics cooling fan	CHECK AUDIBLY

*Avionics master switch	OFF

*Lights (Nav/Strobe/Beacon/Landing/Taxi)	ONCHECKOFF

*Pitot heat (pitot tube warn within 30 sec)	ONCHECKOFF

*Flaps	EXTEND FULL

Annunciator lights (hold test switch)	CHECK

Master switch	OFF

Flight controls and trim	CHECK FULL MOVEMENT

Engine controls	CHECK FULL MOVEMENT

Seats and harness	CHECK CONDITION

Windows clean (see note)	CHECK

NOTE: To avoid damage/scratches on the windows, use dedicated cleaning equipment only.

C. LEFT WING

Surface condition	CHECK

Flap and hinges	CHECK

Aileron and hinges	CHECK

Wing tip	CHECK

Lights	CHECK

Pitot head (unobstructed)	CHECK

Stall warning	CHECK

Fuel cap	OPEN

Fuel quantity and color	CHECK

Fuel cap	CLOSE AND SECURE

Fuel vent (unobstructed)	CHECK

Landing gear and tire	CHECK

Brake block and disc	CHECK

Chocks and tie down	REMOVED

D. NOSE SECTION

General condition	CHECK

Static pressure port (unobstructed)	CHECK

Propeller and spinner	CHECK

Air inlets/filter (unobstructed)	CHECK

Oil (see note)	CHECK

Dipstick	PROPERY SEATED

Nose wheel tire	CHECK

Nose gear strut (proper inflation 8-9 cm)	CHECK

NOTE: Factory approved oil level is between 4 and 9 quarts. Try to maintain oil level between 6 and 8 quarts. Never overfill. Always use a funnel when filling oil. Use Total Aero DM 15W50 oil all year (requires engine heater during winter).

E. RIGHT WING

Check as left wing.

(Pitot head and stall warning are on the left wing only)

F. FUSELAGE, RIGHT SIDE

General condition	CHECK

Cabin door and hinges	CHECK

Antennas	CHECK

G. EMPENNAGE

General condition	CHECK

Control surfaces	CHECK

Hinges and attachments	CHECK

Antennas	CHECK

Tie down	REMOVED

H. FUSELAGE, LEFT SIDE

General condition	CHECK

Cabin and baggage doors and hinges	CHECK

Antennas	CHECK

I. COCKPIT

Aircraft documents	ON BOARD AND SIGNED

Baggage	STOWED AND SECURED

Passenger briefing (incl. emergencies	PERFORMED

STARTING

1.BEFORE STARTING

Parking brake	SET

Preflight/D.I	COMPLETED/SIGNED

Chocks/Tie downs	.REMOVED

Seats/Harness	ADJUSTED-LOCKED/FASTEN

Doors	LATCHED

Static source	NORMAL

Avionics master/Electrical equipment	OFF

Cowl flap	OPEN

Circuit breakers	CHECK

CAUTION:

The avionics master switch should be OFF during engine

start to prevent possible damage to avionics.

2.STARTING

Throttle	1/4" OPEN

Propeller (push full in)	HIGH RPM

Mixture	IDLE CUT OFF

Master switch	ON

Fuel level	ENOUGH

Fuel selector	BOTH

If engine is cold, prime as follows:

Aux fuel pump	ON

Mixture: advance until fuel flow (5 sec), then	IDLE CUT OFF

Aux fuel pump	OFF

Rotating beacon	ON

Propeller area	CHECK CLEAR

Starter	ENGAGE

Mixture (when engine starts)	FULL RICH

Oil pressure (green arc within 30 - 60 sec)	CHECK

Mixture (7 half turns)	LEAN

3.WARM-UP

Throttle	1000 RPM (winter 1200 RPM)

Engine instruments/Annunciators	CHECK

Avionics master switch	ON

COM/NAV radios/Intercom	SET

Transponder	STANDBY

Elevator trim	SET FOR TAKEOFF

Wing flaps	CHECK EQUAL/RETRACT

External-/Internal lights	AS REQUIRED

4.TAXI

Altimeter (both)	SET

HSI/NAV GPS switch	SLAVE

Taxi light	ON

Taxi area	CLEAR

Brakes	CHECK BOTH SIDES

Nose Wheel Steering	CHECK BOTH SIDES

Slip-/Skid indicator/Directional gyro/Horizon	CHECK

5.GROUND CHECK

Parking brake	SET

Taxi light	OFF

Area	CHECK CLEAR

Mixture	FULL RICH

Throttle	1800 RPM

Engine instruments/Annunciators/Ammeter	CHECK

Magnetos  (Max drop/diff: 150/50 RPM)	R+L CHECK

Propeller	HIGH LOW  HIGH

Vacuum gauge	CHECK

Throttle	slowly to IDLE then 1000 RPM

6.BEFORE TAKEOFF

Flight controls	CHECK

Elevator trim/Electrical trim (check movement)	T/O POS

TAKEOFF

Wing flaps (max 20° for T/O)	CHECK – T/O POS

Mixture (full in)	FULL RICH

Propeller (full in)	HIGH RPM

Engine instruments/Annunciators	CHECK

Flight instruments	CHECK

Strobe lights	ON

Landing light	ON

Pitot heat	ON

T/O briefing	PERFORMED

T/O emergency briefing:

On runway	Throttle OFF/Brakes apply

After airborne	Nose down 75KIAS/Land straight ahead

After 90	Land straight ahead or return to airfield

Before emergency landing	Fuel and switches OFF

Compass/HSI (compass + HSI shows RWY HDG)	CHECK

Transponder	ALTUTUDE

Autopilot (control wheel moving freely)	OFF

Time	NOTE

7.TAKEOFF

Power	APPLY FULL

Power static output (abort if to low)	min 2400 RPM

Engine instruments	NORMAL

Airspeed (normal T/O: rotate at 55 KIAS)	INCREASING

8.CLIMB

Climb speed	85 - 95 KIAS

Power settings

Takeoff	25”/2500

Enroute	23”/2400

Cowl flap	OPEN

Fuel selector	BOTH

Wing flaps (+500’ AGL and +70 KIAS)	RETRACT

9.CRUICE

Power settings at standard temperature and 4000’ AMSL:

75 % power	23”/2400

65 % power	22”/2300

55 % power	21”/2200

Landing light	OFF

Mixture (9 half turns = 13.5 GPH)	LEAN

Cowl flap	CLOSE

10.BEFORE AIRWORK

Loose items	STOW

Mixture	FULL RICH

Cowl flap	OPEN

Propeller (full in)	HIGH RPM

Engine instruments	CHECK

Autopilot	CHECK OFF

Harness	CHECK FASTENED

Terrain	CHECK OBST./EMERG.FIELD

Populated area	AVOID

Minimum altitude	MIN 2500 AGL / STALL 3000 AGL

Clearance turn	PERFORMED

11.DESCENT

Power	AS DESIRED

Mixture	ADJUST / RICH

Cowl flap	CLOSED

Fuel selector	BOTH

HSI/NAV GPS switch	SLAVE

Wing flaps (check speed)	AS DESIRED

12.BEFORE LANDING

LANDING

APPROACH:

Loose items / Harness / Seat backs	CHECK

Altimeter (both)	SET

HSI/NAV GPS switch	SLAVE

Landing light	ON

GPS/NAV switch	NAV

Autopilot	OFF

DOWNWIND:

Magnetos	BOTH

Mixture	FULL RICH

Fuel selector	BOTH

Approach speed	75 KIAS / 65 KIAS FULL FLAPS

KEYPOINT:

Wing flaps	AS REQUIRED

Propeller (full in)	HIGH RPM

Cowl flap (to prepare for go around)	OPEN

13.ABORTED LANDING

Power (all full in)	APPLY FULL

Wing flaps	RETRACT TO 20°

Climb speed	55 KIAS

Wing flaps (+500’ AGL and +70 KIAS)	RETRACT

13.TOUCH AND GO

Wing flaps (max 20°)	retract to T/O POSISTION

Elevator trim	T/O POSITION

Power (all full in)	APPLY FULL

Power static output (abort if to low)	min 2400 RPM

Airspeed (normal T/O: rotate at 55 KIAS)	INCREASING

14.AFTER LANDING

Wing flaps	RETRACT

Landing light / Taxi light	OFF / AS REQUIRED

Strobe lights (when clear of RWY)	OFF

Pitot heat	OFF

Transponder	7000 SET / STANDBY

Cowl flap	OPEN

Time	NOTE

15.SHUTDOWN

Parking brake	SET

Taxi light	OFF

Throttle	IDLE

Propeller (full in)	HIGH RPM

Magnetos	OFF MOMENTERIYLY, THEN BOTH

Throttle	1000 RPM

Avionics master / Electrical equipment	OFF

Mixture	IDLE CUT-OFF

Magnetos	OFF (keys on glareshield)

Rotating beacon	OFF

Master switch	OFF

Fuel selector (to prevent cross feed)	LEFT or RIGHT

Cabin vents	CLOSE

16.PARKING

Wheel chocks	ON

Parking brake	OFF

Control wheel	SECURED

Tie downs	SECURED

Engine/cabin heater (during winter/cold periods)	CONNECTED

NOTE:

Leave cockpit tidy. Except during the winter season, clean windows and all leading edges for bugs/dirt. Use only dedicated equipment on windows to avoid scratches.

1.ENGINE FAILURE DURING TAKEOFF

On runway:

Throttle	OFF

Brakes	APPLY

Immediately after takeoff:

Safe airspeed	75 KIAS (flaps up) / 70 KIAS (with flaps)

- Land on remaining runway or a suitable area straight ahead.

- Perform relevant parts of emergency landing checklist (no. 3)

2.ENGINE FAILURE IN FLIGHT  (Restart procedures)

Safe airspeed	75 KIAS

Fuel Selector	BOTH

Aux fuel pump	ON

Mixture	FULL RICH

POWER LOSS

Magnetos	BOTH

-If the propeller is not windmilling  Magnetos	START

-If the start attempt is unsuccessful:

Continue with the emergency landing checklist (no. 3)

3.EMERGENCY LANDING (Without engine power)

Most suitable area	LOCATE

Wind direction and effect	CHECK / EVALUATE

Safe airspeed	75 KIAS (flaps up) / 70 KIAS (with flaps)

ELT/Transponder/Radio	ACTIVATE / 7700 SET / MAYDAY

Seats/Seat Backs/Harness	SECURE / UPRIGHT / FASTEN

Mixture	IDLE CUT-OFF

Fuel selector	OFF

Magnetos	OFF

Flaps (full recommended)	AS REQUIRED

Aux fuel pump	OFF

Master switch (when electrical power is no more needed)	OFF

Loose items (or jettison)	SECURE

Emergency equipment	AS REQUIRED

Doors (just prior to touchdown)	UNLATCH

Touchdown	SLIGHTLY TAIL LOW

Brakes	APPLY HEAVILY

Ditching:

High winds, heavy seas	LAND INTO THE WIND

Light winds, heavy swells	PARALLEL TO SWELLS

Rivers	DOWNSTREAM

Face (at touchdown)	CUSHION

4.PRECAUTIONARY LANDING (With remaining engine power)

Locate and inspect suitable field at 75 KIAS and flaps 20°. Evaluate size, wind, surface condition, obstacles and slope.

Autopilot (normally OFF)	CHECK

Loose items / Harness / Seats	CHECK

Lights	AS REQUIRED

Avionics power / Electrical equipment	OFF

Wing flaps	Normally FULL

Airspeed (final)	70 KIAS

Master switch (when electrical power is no more needed)	OFF

Doors (just prior to touchdown)	UNLATCH

Touchdown	SLIGHTLY TAIL LOW

Magnetos	OFF

Brakes	APPLY HEAVILY

5.ABNORMAL OIL PRESSURE OR OIL LEAKAGE

Power	REDUCE

Altitude	CONSERVE

Cowl flap	OPEN

Engine instruments	MONITOR CONTINOUSLY

- Divert to nearest airport

- Prepare for emergency landing (no. 3)

6.ENGINE FIRE DURING START

Starter (to suck out flames and fuel)	CONTINUE CRANKING

If engine starts:

Power (for a few minutes)	1700 RPM

Engine (inspect for damage)	SHUTDOWN

If engine fails to start:

Starter 	 CONTINUE CRANKING

Throttle (full in) 	FULL OPEN

Mixture (full out)	 IDLE CUT OFF

Fuel selector	 OFF

Aux fuel pump	 OFF

Master switch	OFF

Magnetos	 … OFF

Parking brake 	 RELEASE

EVACAUTE AIRCRAFT AND EXTINGUISH FIRE

7.ENGINE FIRE IN FLIGHT

FIRE

Mixture	IDLE CUT -OFF

Fuel selector 	OFF

Aux fuel pump	OFF

Master switch	OFF

Cabin heat and air (except overhead vents)	OFF

Airspeed 	 100 KIAS

If fire is not extinguished, increase glide speed to find an airspeed – within airspeed limitations – which will provide an incombustible mixture.

Forced landing (emergence landing checklist - no. 3) 	EXECUTE

8.ELECTRICAL FIRE IN FLIGHT

Master switch	OFF

Vents / Cabin air / Heat	CLOSED

Fire extinguisher (if available)	ACTIVATE

Avionics master switch	OFF

All other switches (except magnetos) 	OFF

Vents / Cabin air / Heat (to ventilate cabin)	OPEN

If fire has been extinguished and electrical power is necessary for continuance of flight to nearest suitable airport or landing area:

Master switch	ON

Circuit breakers (do not reset)	CHECK

Radio switches	OFF

Avionics master switch 	ON

Radio / Electrical switch (one at a time)	ON

9.CABIN FIRE

Master switch	OFF

Vents / Cabin air / Heat	CLOSED

Fire extinguisher	ACTIVATE

Vents / Cabin air / Heat (when fire is extinguished)	OPEN

Wing fire (see note):

External lights	OFF

Pitot heat	OFF

NOTE:

Perform a sideslip to keep the flames away from the fuel tank and cabin. Use flaps only as required for final approach and touchdown.

Land as soon as possible

10.ICING

Pitot heat 	ON

Turn back or change altitude to obtain an outside air temperature that is less conducive to icing.

Cabin heat / Defroster	MAX

Airspeed	INCREASE

Watch for signs of induction air filter icing. An unexplained loss of manifold pressure could be caused by ice blocking the air intake filter.

Throttle (for desired manifold pressure)	ADJUST

Mixture (after change in power setting)	ADJUST

Wing flaps	RETRACTED

Land	NEAREST AIRPORT or OFF AIRPORT

Windshield (if possible, for visibility)	SCRAPE FOR ICE

Approach (for visibility)	FORWARD SLIP

Approach speed (depending upon amount of ice)	80 to 90 KIAS

Land	LEVEL ATTITUDE

11.STATIC SOURCE BLOCKAGE

ICING

Alternate static source	ON

Airspeed 	 Consult appropriate calibration table

Altitude 	 Consult altimeter correction table

12.LANDING WITH A FLAT MAIN TIRE

Approach	NORMAL

Wing flaps	FULL DOWN

Touchdown 	 GOOD MAIN TIRE FIRST

Directional control (using brake on good tire) 	MAINTAIN

Keep flat tire in the air as long as possible using aileron control.

13.LANDING WITH A FLAT NOSE TIRE

Approach	NORMAL

Wing flaps	AS REQUIRED

Touchdown 	ON MAIN TIRES

Hold nose wheel of the ground as long as possible. When nose wheel touches down, maintain full up elevator as airplane slows to stop.

14.AMMETER SHOWS EXCESSIVE RATE OF CHARGE

Alternator 	 OFF

Nonessential electrical equipment	OFF

Flight (as soon as practical)	TERMINATE

15.LOW VOLTAGE WARNING (AMMETER DISCHARGE)

Avionics master switch	OFF

Alternator circuit breaker	CHECH IN

Master switch (both sides)	OFF

Master switch	ON

Low voltage annunciator	CHECK OFF

Avionics master switch	ON

If low voltage light illuminates again:

Alternator	OFF

Nonessential radio and electrical equipment	OFF

Flight (as soon as practical)	TERMINATE

16.VACUUM SYSTEM FAILURE

Left or right vacuum annunciator light illuminates (L VAC R).

Caution!

If vacuum is not within normal operating limits, a failure has occurred in the vacuum system and partial panel procedures may be required for continued flight.

Suction gage 	CHECH

W & B CALCULATION SHEET (metric)

| Reg. Appl.: LN-EDB (G-IBZS) |  |  |  |
| --- | --- | --- | --- |
| Load type | Weight(kg) | Arm(mm) | Moment(kg-mm) |
| Empty weight | 899 | 947 | 851 506 |
| Pilot + front passenger |  | 940 * |  |
| Rear passenger(s) |  | 1880 |  |
| Fuel (max 333 ltr) |  | 1181 |  |
| Baggage 1 (max 54 kg) ** |  | 2464 |  |
| Baggage 2 (max 23 kg) ** |  | 2946 |  |
| Baggage 3 (max 36 kg) ** |  | 3277 |  |
| Total (max load 1406 kg) |  |  |  |

| Total moment | / Total weight | = Arm (mm aft of datum) |
| --- | --- | --- |

* Forward seats adjustable from 813 mm to 1270 mm CG arm.

** Max. total baggage weight, all compartments: 90 kg.